GreenPulse: Clean Construction Power System

Sime Darby Property University Hackathon 2026 (Planet Track) — Master System Guide

1. Executive Summary

Core Concept: GreenPulse is an intelligent clean energy management system for construction sites that completely replaces noisy, expensive diesel generators with Solar Panels + Mobile Battery Storage (BESS) + EV Machinery, all coordinated by our proprietary Smart Energy Software.

2. The Problem Facing Sime Darby Property Today

When Sime Darby develops a new township parcel (such as City of Elmina, Bandar Bukit Raja, or Putra Heights), site development faces four immediate operational bottlenecks:

1. No Grid Power During Critical Construction Phases: Permanent TNB electrical substations (Pencawang Elektrik) are only commissioned near building completion (which takes 2 to 4 years in Malaysia). Meanwhile, securing temporary TNB supply takes 4 to 9 months, or is completely unavailable in greenfield township parcels (as documented at Rimbayu where there is zero TNB supply). Furthermore, temporary supplies cannot handle the heavy power surges of tower cranes.

2. Expensive Diesel Fuel (Over RM 62,000 / Month per Site): Because there is no grid power, sites rely on large 500 kVA diesel generators running 24/7. As of October 2026 (official Setel / PETRONAS weekly rate), unsubsidised commercial diesel pump price is RM 5.17 / litre (Euro 5) and RM 5.37 / litre (B7). Commercial construction equipment is strictly excluded from the subsidised BUDI rate (RM 2.10/L). Burning 12,000 litres of diesel at RM 5.17/L costs over RM 62,000 every single month on just fuel.

3. Severe Noise Pollution: Diesel generators run continuously, generating heavy noise pollution. This frequently triggers resident complaints to the Department of Environment (DOE), risking work stop orders and project delay penalties.

4. Heavy Carbon Footprint: Burning thousands of litres of diesel releases substantial carbon emissions (approx. 12 to 32+ tonnes of CO2 per month depending on site size), directly conflicting with Sime Darby Property's corporate 40% carbon reduction targets.

3. Our Solution: The GreenPulse System

We introduce a complete zero-diesel power package to Sime Darby Property:

A. The Clean Hardware Package

Rooftop Solar Panels: Installed on workers' quarters and site offices to generate free daytime electricity.

Mobile Battery Container (BESS): A containerised modular battery (261 kWh to 522 kWh capacity, as proven on actual sites) that stores solar power and delivers clean power 24/7.

Electric Construction Machinery (EVs): Battery-powered forklifts, electric hoists, and electric tower cranes to eliminate diesel engines on site.

B. The Ground Reality: What Exists on IESS Cloud Today

On actual construction sites today (such as Inta Bina Putra Heights with 0.261 MWh and Inta Bina Melati with 0.522 MWh), the only software currently deployed is IESS Cloud.

As seen on their live dashboard, IESS Cloud only shows static hardware stats: Installed Capacity (0.261 MWh or 0.522 MWh), Installed Power (0.2 MW or 0.4 MW), and an 'Online' status dot.

Why Existing IESS Cloud Software Falls Short: • Blind to Weather: No connection to weather radar or solar irradiance forecasting. • Blind to Crane Surges: No sub-second inrush buffering to prevent power trips when lifting heavy concrete. • No EV Scheduling: No schedule for charging electric forklifts and hoists during peak solar. • No Financial or ESG Tracking: No calculation of diesel fuel saved, money saved, or carbon emissions cut.

C. The Smart Energy Software (The Brain We Introduce)

Hardware alone is not enough. Without intelligent management, crane startup spikes can trip the battery, and rain can leave the site without power. GreenPulse provides the missing software layer:

Weather Forecasting: Connects to local weather radar. If rain is forecast, it pre-charges the battery during sunny hours so work never stops.

Crane Inrush Buffer: Delivers an instant power boost when the 50-tonne crane lifts heavy concrete, preventing power trips.

Smart EV Charging: Schedules electric forklifts and hoists to charge during peak midday sunshine when solar energy is free.

Automated Savings Ledger: Tracks diesel litres eliminated, money saved, and verifies that site noise remains quiet.

4. The Two Target Users

The platform serves two primary stakeholders with tailored operational and executive views:

| Dimension | User 1: On-Site SHE & M&E Officer | User 2: Sime Darby HQ Management |
| --- | --- | --- |
| Location & Device | On the Construction Site (iPad / Rugged Field Laptop) | Sime Darby Property HQ (Executive Desktop / Tablet) |
| Primary Role | Site Safety, Health & Environment (SHE) / M&E Engineer | Township Project Director / Sustainability & ESG Lead |
| Core Focus | Ground execution, zero equipment trips, noise compliance | Cost savings (P&L), ESG reporting, asset relocation |
| Daily Checklist | • Check current & tomorrow's weather forecast • Verify Battery SoC % and power output • Monitor equipment power flows (Crane, Hoist, EV) • Confirm standby diesel generator is 100% OFF • Keep site boundary noise below quiet threshold | • Total diesel expenditure saved vs baseline (RM) • Litres of diesel fuel displaced across parcels • Verified Scope 1 & 2 carbon emissions avoided (tCO2e) • Review equipment relocation timeline across parcels |

5. What Happens When Construction Finishes?

100% Modular and Mobile Asset Lifecycle: In Malaysia, a housing development or high-rise takes approximately 2 to 3+ years (24 to 36+ months). Once structural construction reaches completion and permanent TNB grid power is energized, the containerised battery and solar units are disconnected, loaded onto a flatbed trailer, and transported 20 minutes down the road to Sime Darby's next township parcel. Nothing is stranded, and capital investment continues yielding returns over a 10-year lifespan.

6. What Our Prototype Shows (Two Clean Core Screens)

Screen 1: Live Site Energy & Weather Operations (On-Site View)

Live Weather & Predictive Card: Displays current meteorological conditions and tomorrow's 24-hour solar forecast (e.g., sunny morning vs afternoon monsoon storm).

Battery Telemetry Status: Live State of Charge (SoC %), battery temperature, voltage, and health status.

Site Power Flow Distribution: Real-time kilowatt distribution to the 50t Tower Crane, Passenger Hoists, EV Forklifts, and Site Office.

Zero-Diesel Guarantee Indicator: Live visual confirmation badge showing the standby diesel generator is 100% OFF.

Screen 2: Financial & Carbon Savings (HQ Executive View)

Live Weekly Malaysian Diesel Benchmark Tracker: Displays official weekly unsubsidised market fuel prices (e.g., PETRONAS Dynamic Diesel Euro 5: RM 5.17 / L, Diesel B7: RM 5.37 / L for 15–21 Oct 2026). Automatically ties into savings formulas: Litres Displaced × Current Fuel Price = Real-Time RM Saved.

Visual Impact KPI Cards (As seen on live systems):

CO2 Emission Reduction: Direct metric in Tonnes (e.g., 84.15 Tonnes avoided).

Diesel & Fuel Cost Saved: Total litres avoided and Ringgit saved (over RM 62,000 / month per site).

Equivalent Trees Planted: Intuitive ESG translation metric (e.g., 575 Trees Planted).

Noise Compliance Meter: Real-time noise status showing site boundary noise is quiet and strictly compliant with DOE residential guidelines.

Bursa Malaysia ESG Summary Export: One-click audit table ready for corporate sustainability disclosures.

7. Reference Guide: The Five Energy Operating Tiers

GreenPulse dynamically categorizes the 24-hour daily cycle into five distinct operating tiers to orchestrate battery charging, heavy machinery scheduling, and surge buffering:

| Energy Tier | What It Means on Site | Automated System Action |
| --- | --- | --- |
| Critical Peak (Red) | Maximum Power Surge: Tower crane and passenger hoists lift heavy loads simultaneously; site demand peaks. | Battery discharges at maximum power to buffer the spike in milliseconds, preventing breaker trips and eliminating generator startup. |
| Peak (Orange) | High Work Shift (09:00 - 17:00): Full construction activity with hoists, bar benders, lighting, and site offices running. | Battery delivers clean stored energy to cover all active operational loads while the standby diesel generator remains 100% OFF. |
| Shoulder (Cyan) | Transition Hours (07:00 - 08:30 & 17:00 - 19:00): Shifts beginning or winding down; machinery power demands moderate. | System balances state of charge, prepares evening schedule, and diagnostics verify zero system faults. |
| Off-Peak (Green) | Nighttime Baseline (20:00 - 07:00): Heavy machinery idle; only perimeter security lighting and worker ventilation running. | Low, steady battery discharge. Zero engine noise so surrounding residential neighbours enjoy peaceful nights. |
| Deep Off-Peak (Purple) | Golden Solar Window (11:30 - 14:30): Midday solar generation reaches maximum peak surplus. | Maximum solar charging into the 522 kWh BESS; electric forklifts and EV machinery are scheduled to plug in for 100% free charging. |